Measurement Name

DRAlogs_SummaryTrace (accessed via "1hrRP" retention policy)

Description

This measurement captures individual Diameter transaction events passing through the DRA. Unlike the "KPI" table which aggregates counts, this table records individual request outcomes, allowing you to trace specific routing flows (e.g., 3GPP-Gx) and their specific result codes (e.g., DIAMETER_SUCCESS). It is essential for troubleshooting specific connection failures or protocol errors.

1. Schema Breakdown

Tags (Dimensions)

Use these columns to filter traffic by protocol result or routing path.

Tag Key

Description

Example Value

ResultCode

[Critical] The Diameter protocol result code. Used to calculate success rates (2xxx) vs. failures (3xxx/5xxx).

DIAMETER_SUCCESS

FlowName

The specific 3GPP interface being routed.

3GPP-Gx

Type

The internal routing logic or rule applied.

gxgy_routing_2

host

The specific OCS or DRA node handling the traffic.

DLVMAMXOCS01

zone

The geographical zone.

Dallas_North

project

The project identifier.

AMX

service

The service context.

dra

nodeIP

The IP address of the node.

10.221.87.165

path

The log file source.

/.../logs/dra/SummaryTrace-DRA.csv

Export to Sheets

Fields (Metrics & Attributes)

These columns contain the raw event data.

Field Key

Type

Description

timestamp

String

The original string timestamp from the log file.

millisecond

Integer

The sub-second (millisecond) component of the event time.

Export to Sheets

2. Sample Queries

A. Calculate Error Rates (Success vs. Failure)

Group by ResultCode to see the ratio of successful transactions vs. errors.

SQL

SELECT COUNT("millisecond")

FROM "1hrRP"."DRAlogs_SummaryTrace"

WHERE time > now() - 1h

GROUP BY "ResultCode"

B. Analyze Traffic by Interface (Flow)

See which Diameter interfaces (Gx, Gy, Rx, etc.) are generating the most logs.

SQL

SELECT COUNT("millisecond")

FROM "1hrRP"."DRAlogs_SummaryTrace"

WHERE time > now() - 6h

GROUP BY "FlowName"

C. Investigate Specific Routing Rules

Check if a specific routing rule (gxgy_routing_2) is causing issues.

SQL

SELECT "ResultCode", "host", "timestamp"

FROM "1hrRP"."DRAlogs_SummaryTrace"

WHERE "Type" = 'gxgy_routing_2'

AND "ResultCode" != 'DIAMETER_SUCCESS'

LIMIT 20